## Cleaning Hot Tubs

### Purpose

This Safe Work Practice explains how to safely clean, maintain, and refill a hot tub

###### This procedure supports employer and worker duties under the Workers Compensation Act.

### Know Your Responsibilities

##### Employers Must

- Provide a safe workplace and equipment

- Identify, assess, and control hazards

- Train and supervise workers in safe practices

- Maintain SWPs, emergency procedures, and safety policies

- Investigate and report incidents

##### Workers Must

- Follow safe work practices and procedures

- Wear and use required PPE

- Report hazards or unsafe conditions

- Participate in training and drills

- Work safely to protect self and others

### What Could Go Wrong?

- Chemical burns or irritation from cleaning products

- Eye injury from splashes or fumes

- Slips from wet floors

- Exposure to chemical vapors or concentrated cleaners

### Before You Start

- Be trained and authorized

- Take time to assess the situation and plan your approach

- Close the hot tub to guests

- Confirm all cleaning chemicals and PPE are ready

- Ensure good ventilation in the hot tub area

- Clear floors and remove obstacles around the tub

### Required PPE

- Respirator mask

- Eye protection

- Rubber gloves

- Non-slip footwear

### Safe Work Steps

- Close the hot tub to guests

- Close the steam valve carefully

- Drain the hot tub completely following manufacturer's instructions

- Put on personal protective equipment

- Follow cleaning steps according to product instructions

- Rinse the walls, tiles, and corners with cold running water for 5 minutes

- Clean up any leftover water on the floor and under mats

- Remove hair and lint from the hot tub grates and the skimmer

- Dry the hot tub with a towel

- Fill the hot tub with hot running water and add the necessary chemicals

- Close all valves and turn on the pumps

- Let the hot tub run for three hours, then test the water and adjust chemicals if necessary

- Once the chemical balance is correct, reopen the hot tub for guest use

- Record details of chemical use in the workplace logs

### Special Considerations

- Ensure proper ventilation when using chemicals

- Follow WHMIS requirements and manufacturer instructions for all cleaning products

- Do not allow guests in the hot tub until cleaning is complete and chemical levels are correct

### In Case of Emergency

- Report hazards, incidents, and near misses immediately to your supervisor, Joint Health and Safety Committee, or Worker Health and Safety Representative (if applicable)

- Get first aid immediately for cuts or injuries.

- For chemical splashes in the eyes, rinse immediately with water and seek medical attention if needed

- Follow all first aid instructions provided on the Safety Data Sheet (SDS)

### Additional Safety Resources

- WorkSafeBC Chloramines: Safe Work Practices http://www.worksafebc.com/resources/health-safety/books-guides/chloramines-safe-work-practices?lang=en

- WorkSafeBC WHMIS Information https://www.worksafebc.com/en/health-safety/hazards-exposures/whmis

- Swimming pool and spa chemicals https://www.canada.ca/en/health-canada/services/home-garden-safety/swimming-pool-spa-cleaning.html

Date of Release:      
Date of Review:                                                    Reviewed by: