## Fueling Helicopters

### Purpose

This Safe Work Practice explains how to safely refuel helicopters

###### This procedure supports employer and worker duties under the Workers Compensation Act.

### Know Your Responsibilities

##### Employers Must

- Provide a safe workplace and equipment

- Identify, assess, and control hazards

- Train and supervise workers in safe practices

- Maintain SWPs, emergency procedures, and safety policies

- Investigate and report incidents

##### Workers Must

- Follow safe work practices and procedures

- Wear and use required PPE

- Report hazards or unsafe conditions

- Participate in training and drills

- Work safely to protect self and others

### What Could Go Wrong?

- Fire or explosion from fuel vapours

- Chemical exposure from Jet-B fuel

- Musculoskeletal injuries from lifting fuel hoses or equipment

- Spills causing environmental contamination

### Before You Start

- Be trained and authorized

- Take time to assess the situation and plan your approach

- Complete a practicum with a Senior Guide before working alone

- Ensure no guests are near the aircraft

- Inspect fuel hoses, pump, and tank for damage or leaks

### Required PPE

- Clothing to protect against cold and chemical exposure

- Additional PPE required by the hazards of the work activity

### Safe Work Steps

##### Refueling Helicopters

- Always ground the helicopter first

- Turn the fuel valve on

- Start the pump, ensuring hoses have no kinks

- Remove the fuel cap low to the ground; watch for spills

- Insert the nozzle fully into the filler neck; do not remove until finished

- Keep hands on the nozzle; do not multi-task

- Watch for signals to stop fueling

- Ensure the lever is fully down, not caught on a notch

- Lift hose above nozzle to drain fuel from spout

- Close and lock the fuel cap

- Remove grounding wire

- Replace hose, shut off tank, shut down pump, latch lid

- Re-check the fuel cap

- Report all spills to the Operations Manager

- Check the pump for fuel; do not rev the engine

- Dip the tank to confirm fuel level

##### Fuel Spill Contingency Plan

- Ensure safety of all guests and staff; move them to a safe location

- If safe, contain the spill:
 • Release the fuel nozzle completely
 • Shut off pump and tank valve
 • Apply absorbent materials
 • Create dams if possible

- Notify immediately: Base radio operator, Operations Manager, Lead Guide

- Radio operator stays by radio and fills out Spill Report Form

- For spills >100 litres (22 US gal), notify Provincial Emergency Program at 1-800-663-3456

- Protect yourself with gloves and avoid fumes or fire

- Call 911 for injuries, fire, or public safety hazards

- Evacuate base if spill is uncontrollable; notify Fire Department immediately

### Special Considerations

  - Fuel vapours are highly volatile; never rush the process

  - Always maintain communication with the flight crew

  - Never refuel with guests nearby

### In Case of Emergency

- Report hazards, incidents, and near misses immediately to your supervisor, Joint Health and Safety Committee, or Worker Health and Safety Representative (if applicable)

- Get first aid immediately for cuts or injuries

- <list additional emergency steps>

### Additional Safety Resources

- Part 5 of the OH&S Regulation - https://www.worksafebc.com/en/law-policy/occupational-health-safety/searchable-ohs-regulation/ohs-regulation/part-05-chemical-and-biological-substances

- CCOHS Fuel Safety – https://www.ccohs.ca/oshanswers/safety_haz/garages/fuel.html

Date of Release:      
Date of Review:                                                    Reviewed by: