## Loading and Unloading Helicopter Baskets

### Purpose

This Safe Work Practice explains how to safely load and unload equipment into a helicopter basket

###### This procedure supports employer and worker duties under the Workers Compensation Act.

### Know Your Responsibilities

##### Employers Must

- Provide a safe workplace and equipment

- Identify, assess, and control hazards

- Train and supervise workers in safe practices

- Maintain SWPs, emergency procedures, and safety policies

- Investigate and report incidents

##### Workers Must

- Follow safe work practices and procedures

- Wear and use required PPE

- Report hazards or unsafe conditions

- Participate in training and drills

- Work safely to protect self and others

### What Could Go Wrong?

- Strains from lifting or carrying heavy equipment

- Falls from slips or trips on snow, ice, or uneven ground

- Injuries from rotor wash, noise, or flying debris

- Injuries from loose items being pulled into the helicopter

### Before You Start

- Be trained and authorized

- Take time to assess the situation and plan your approach

- Confirm roles before loading or unloading equipment

- Plan who will communicate with the pilot, hold the door, and carry equipment

- Prepare a landing pad area to use as a work site

- Ensure the area is clear of loose clothing, plastic bags, or debris

### Required PPE

- High visibility apparel

- Hearing protection

- Eye protection

- Dust mask as required

### Safe Work Steps

##### Bundling Skis

##### Powder straps

- Twist both straps together in the same direction around both skis between the toe and heel of the binding

- Drag skis by the tips

##### With wrist straps

- Place one strap on each ski tip

- Twist the straps together tightly

- Place a Velcro ski strap between the toe and heel bindings through the buckle and secure tightly

- Ensure the straps are tight before loading or unloading

- Drag skis by the tips

##### Without wrist straps

- Turn poles upside down and place baskets between the ski tips

- Use two straps. One near the ski tips and one between the toe and heel of the binding

##### Loading Equipment

- Pack a landing pad to create a stable work area

- Stack skis in pairs beside the landing pad

- Knock excess snow off skis before stacking

- Wait until helicopter rotors are shut down before loading

- Do not lift skis above your head around the helicopter

- Lock the ski box open with the locking mechanism

- Lift one pair of skis at a time into the ski basket

- Use proper lifting techniques when lifting equipment

- Close and lock the basket carefully

##### Unloading Equipment

- The guide will open and close the ski basket

- Maintain visual contact with your helper

- Ensure stable footing before lifting

- One person holds the ski tips and the other holds the tails

- Lift bundled skis out of the basket using proper lifting techniques

- Keep skis horizontal when removing them to prevent them from lifting toward the rotor

### Special Considerations

  - Helicopter areas are noisy and windy due to rotor wash
Clear communication and planning are important to prevent confusion and accidents

### In Case of Emergency

- Report hazards, incidents, and near misses immediately to your supervisor, Joint Health and Safety Committee, or Worker Health and Safety Representative (if applicable)

- Get first aid immediately for cuts or injuries

- Follow pilot or guide instructions during any helicopter emergency

### Additional Safety Resources

- WorkSafeBC OHS Guidelines Part 29 – Aircraft Operations https://www.worksafebc.com/en/law-policy/occupational-health-safety/searchable-ohs-regulation/ohs-guidelines/guidelines-part-29

- Helicat Canada Safety https://www.helicat.org/safety

Date of Release:      
Date of Review:                                                    Reviewed by: