## Loading and Unloading Snowcat Baskets

### Purpose

This Safe Work Practice explains how to safely load and unload equipment, skis, and snowboards into a snowcat basket

###### This procedure supports employer and worker duties under the Workers Compensation Act.

### Know Your Responsibilities

##### Employers Must

- Provide a safe workplace and equipment

- Identify, assess, and control hazards

- Train and supervise workers in safe practices

- Maintain SWPs, emergency procedures, and safety policies

- Investigate and report incidents

##### Workers Must

- Follow safe work practices and procedures

- Wear and use required PPE

- Report hazards or unsafe conditions

- Participate in training and drills

- Work safely to protect self and others

### What Could Go Wrong?

- Musculoskeletal injuries from lifting or twisting

- Slips, trips, or falls around the snowcat

- Injuries from falling equipment if straps or storage are not secure

- Confusion or accidents if communication with the operator is unclear

### Before You Start

- Be trained and authorized

- Take time to assess the situation and plan your approach

- Confirm roles: who communicates with the operator and who handles equipment

- Ensure stable footing around the machine

- Check all straps and storage compartments before loading/unloading

### Required PPE

- High visibility apparel

- Hearing protection

- Eye protection

- Any additional PPE required by the hazards of the work activity

### Safe Work Steps

##### Bundling Skis

###### Powder Straps

- Twist both straps in the same direction around both skis between toe and heel of the binding

###### With Wrist Straps

- Place one strap on each ski tip

- Twist straps tightly together

- Place Velcro ski strap between toe and heel bindings through the buckle and secure

- Always drag skis by the tips

###### Without Wrist Straps

- Turn poles upside down and nest baskets between the ski tips

- Use two straps: one near the tips and one between toe and heel bindings

##### Loading/Unloading Equipment

- The guide opens and closes the storage caddy

- Always keep visual contact with your helper

- Maintain stable footing around the machine

- When lifting skis or snowboards, one person holds the tips and the other holds the tails

- Always use proper lifting techniques

- Load and unload equipment from the back, side, or roof storage carefully

- Ensure all straps are secure and locked before the snowcat moves

- Be aware that snowcat vibrations can dislodge improperly secured equipment

### Special Considerations

  - Never toss items from person to person

  - Communicate clearly with helpers and operator

  - Check roof and storage caddies for snow and ice that may affect stability

### In Case of Emergency

- Report hazards, incidents, and near misses immediately to your supervisor, Joint Health and Safety Committee, or Worker Health and Safety Representative (if applicable)

- Get first aid immediately for cuts or injuries

- Follow workplace emergency procedures if someone is injured or pinned

### Additional Safety Resources

- WorkSafeBC OHS Regulation Part 16 – Mobile Equipment https://www.worksafebc.com/en/law-policy/occupational-health-safety/searchable-ohs-regulation/ohs-regulation/part-16-mobile-equipment

Date of Release:      
Date of Review:                                                    Reviewed by: